User Instructions:

- Check to make sure DHCP server is running by looking for devices on network. Reboot if needed by holding down power for 8 seconds and then powering up after it turns off.

- Remote into the VXI computer at 192.168.1.207 and make sure Resource Manager has allocated the VXI memory addresses (from inside NI-Max). This computer sometimes locks up and has to be restarted and then Resource Manager run.

- Remote into the Matisse computer at 192.168.1.1XX and make sure the Sirah laser, the wavemeter, and the PID controller are running

- Make sure the X: drive is mounted on the lab PC

- Make sure the COM port assignment software is running on the lab PC

- Open valves to gas lines for gas regulator and turn on pressure regulator (turn off at end of day – set to zero)

DAQ development

- Timing info needs to be dumped to D:/PHASMA_RawData - TR

- Finish writing of raw data to MDS and map MDS (Joergers, MSO5104_01, optical tomography. Thomson, Axial B(z), Gun/trigger timing offsets and gun pressure will be added next - TR).

- Test plasma gun motor drives with DAQ system.

- Confirm that trigger fires fast photodiode system and other devices and that the data from the new digitizers is being written to the PHASMA raw data directory.

- Interlock reinstalled for parallel TS and light baffling improved. Beamline is still exposed. Record photodiode signals with Rigol scopes

- Expand timing #1 trigger A for laser warmup to provide a trigger to all three pulsed YAG lasers (TS para, TS perp, PLIF)

- Make sure the LeCroy scope transfers data.?

- Organize the different MatLab/Python codes that will plot data from different diagnostics (Tom is organizing this) (Nowhere near functional yet; need volunteers to help develop submodules such as porting bdot plotting functionality to python. Jscope will work fine in the meantime – TR 1/15/2025)

Development Diary:

Tom, 3/5/2026

Thomson Scattering:
In Thomson Setup Panel, added a timer to periodically refresh Iccd temperature reading. Also added button to close without saving. Having problems with the script refreshing raw data values from more than one place – seems to reset some fields to compile-time setting (Laser Warmup Time, Q-switch delay) every-other time the window is opened. Not fixed.

Plasma Guns / Pressure Regulator:

Write_PlasmaGunData now saves plasma gun settings (current, voltage etc)  to local file system. Pressure todo.

Added callback-driven refresh for monitoring pressure
Added close without saving button so user can peek settings saving anything

In case I broke anything, an old executable is Phasma 2025 DAQ Main.exe. The one with these changes is Phasma 2025 DAQ V2.0.exe.

Tom, 3/6/2026

No changes made to mds or Labwindows. Started implementing new mds tree but rolled all changes back after I discovered an oversight in my code that I need to correct.